CONTENTS

1.0	-	What does the emulator do?				2

2.0	-	About the Z80						3

3.0	-	A look at the emulator					12

3.01	-	Assembling					15

3.02	-	Z80 Runtime					17

4.0	-	Assembler Directives					22

5.0	-	Exercises							25

1.0 What does this emulator do?:

This program was designed to allow you, the user, to learn the fundamentals of low level programming.

By the time that you get to use this program you will already have learnt the basics of CPU operation and will quite likely have used another form of simulator.

This program will give you the opportunity to put into practice concepts that you have learnt as well as introduce you to new ideas.

2.0 About the Z80:

The Z80 is manufactured by Zilog inc. and was first produced in 1977.

The team that designed the i8080, the forerunner to today’s Pentium class CPU’s, also designed the Z80.

Figure 1 Z80 CPU setup.

The Z80 is classed as an 8 bit microprocessor, this means that it can only deal with 8 data bits at anyone time. However as it has internal registers capable of dealing with 16 bit operations it is possible to perform arithmetic on numbers greater then that available to an 8 bit load.

The Z80 comes in a 40 pin package and operates from a single +5V power supply. The only external signal required to make the Z80 operate is a single phase TTL compatible clock signal, typically 4MHz.

As the Z80 features a 16 bit address bus it is possible to access a memory space of 65536 bytes, more commonly known as 64k.

Memory is generally defined as being in pages, in the case of the Z80's 64k space it is visualised as 256 pages with 256 lines of information on each. This gives a model such as the one if figure 2.

Figure 2 View of Z80 memory map

using page model.

The Z80 follows the standard fetch execute cycle which goes like:

1	Fetch data from the memory location pointed to by the program counter and place in the instruction register.

2	Increment the program counter.

3	Decode the command held in the instruction register and perform the action required.

4	Repeat from step 1.

The Z80 Registers:

Figure 3 Z80 Register setup.

The Z80 features a number of registers as shown in figure 3.

Register A:

This register is also known as the accumulator, it was common in the early days of microprocessors to have one main register for the arithmetic as design would be simpler.

Register F:

Also known as the status register. The majority of the 8 bits that make up this register represent an occurrence of an event.

Bit 0 will be set if a carry occurs during an arithmetic operation, this can be either a carry over or a carry under. For example consider the following binary addition.

Figure 4 Register F broken into

it’s 8 bits.

1111 1111

+0000 0001

1  <   0000 0000

A carry was produced as the new number exceeded the maximum value for an 8 bit register, therefore bit 0 of register F would be set to 1.

Bit 1 is non-testable by the programmer, except via clever programming. It is used by the CPU to ascertain whether an addition or subtraction took place.

1 means subtract and 0 means addition.

Bit 2 is dual purpose. It can indicate an even or odd parity or it can show whether an overflow occurred.

When parity is of concern each bit of the selected register or location will be counted and if an even number of bits is the result then parity will be set to 1 otherwise 0.

If the mode of operation is overflow then it is used to show whether the sign of a number has changed. When a number is said to be signed it is in reference to as whether it is a minus or positive number. The sign bit of any number is the most significant bit (MSB), so for example this binary number would be a minus:

An overflow would be generated if these two numbers were added together.

0111 1111	(+127)

+ 0000 0001	(+1)

1000 0000	(-128)

You can see that the above number has become minus. The actual rules that govern the overflow indicator are:

  If there was a carry from bit 6 to bit 7 but no external carry.

- If there is an external carry but no carry from bit 6 to bit 7.

Bit 4 indicates whether a half carry occurred. The half carry indicator is used by the DAA (Decimal Adjust Accumulator) instruction to correct a number that is in BCD (Binary Coded Decimal) format. A half carry is generated by a carry from the lower nibble of the accumulator to the upper.

Eg.

Bit 6 is set to 1 if the result of an operation is 0 otherwise it is set to 0.

Eg.		SUB A	; Subtract the accumulator from itself.

Register A now equals 0

	Bit 6 of F = 1

Bit 7 is used to show the sign of a number after an operation has occurred. See bit 2 for a description of signed numbers. This bit also plays an important part when used in conjunction I/O devices. As this implementation of the emulator does not support I/O devices it will not be discussed.

It should be noted that bits 3 and 5 are not used.

Collectively all of these bits are known as flags.

Registers B to L:

These are general purpose 8 bit registers. As stated earlier they are not used for arithmetic operations except when used in conjunction with the accumulator.

Eg.		SUB L	; Subtract register L from the accumulator.

It should be noted that register B is also used as a counter for some instructions.

Eg.			LD B,10h		;Load register B with the hex value 10

LOOP:

DJNZ,LOOP	; Subtract 1 from B and if not 0 jump

; back to point LOOP.

HALT		; Stop emulator.

These registers may be joined together to form 16 bit registers, although this can occur in the following order only.

	B and C

	D and E

	H and L

It is important to remember that when you are using, for example, B and C in 16 bit mode then you are in fact using two registers. So any attempt to use either of these two registers will result in a change of the 16 bit value.

In figure 3 there is a collection of registers that mirrors the A - L register set, except for the ‘ symbol.

This register set is known as the alternative register set. The registers in the alternate set perform exactly the same functions as the primary set. The Z80 can only use one of the sets at a time though. This in effect means that they can be viewed as a buffer.

Eg.	If register A = 5 and all other registers = 0 including the alternate set.

EXX AF,AF’	; Change the registers in use, in this case A and F,

; with there alternate counterparts.

A now equals 0 as does F

  A’ = 5 and F’ = 0

Register SP:

This register is known as the stack pointer and is 16 bits wide. The SP register is used to point to the location in memory that holds the last item of data to be entered on the stack.

Eg.

So assuming that SP was given the value of 0000h and that the following registers have the following values.

Register B 20h

Register C 30h

Register D 40h

Register E 17h

Register H 25h

Register L 96h

If we executed the command PUSH BC, then the value in register B would be copied to the location of SP - 1 (Notice how the SP register rotates to 0000h) and the value in register C would be copied to SP - 2. SP would now point to FFFEh. If we then did the same for register pairs DE and HL we would end up with the memory model listed previously, SP would finish up pointing to FFFAh.

The stack is used as a temporary holding area for data and grows downwards in memory as more data is placed on it.

When the data is retrieved via the POP command the chosen register(s) are loaded with the value pointed to by SP and SP + 1. This would leave SP pointing to the address FFFCh of the previous example.

Register PC:

This is a 16 bit wide register and points to the location in memory where the next operator/operand is to be fetched from.

Registers IX and IY:

These are provided to allow indexing to take place, hence the I in the register name.

Indexed addressing is achieved by having a register pointing to a table of data, this data can be read from/written to by adding an offset to the chosen index register.

Eg.

By executing the command LD A,(IX+3) we would copy the value at the location 0003h into the accumulator.

The index register will still point to address 0000h.

Register R:

In a normal Z80 setup this register would be used to provide a refresh signal to DRAM type memory.

This type of signal would be required to prevent any data being stored from becoming erased. The R register would normally output an increasing value up to 7 bits long during certain cycles of the Z80.

This version of the emulator does not generate the signal on the R register as it is not necessary at this point to do so, register R is available apart from that point.

Register I:

Is used for certain types of interrupt operations. This version of the emulator does not support interrupts although the register is active.

Modes of Address:

The Z80 supports the following modes of addressing.

Direct:

Sample mnemonic:	LD A,(FECAh)

This means load the accumulator with the value held at memory location FECAh.

Register:

Sample mnemonic:	LD A,D

This will load the accumulator with the value held by the D register.

Register Indirect:

Sample mnemonic:	LD A,(HL)

This will load the accumulator with the value held at the location pointed to by register pair HL.

Immediate:

Sample mnemonic:	LD A,50h

This will load the accumulator with the value of 50h.

Relative Branching:

Sample mnemonic:	JR 20h

This will move the PC by the value of 20h. Relative type instructions use two’s compliment to compute the new address. This means that a relative type jump can be in the range of +127 (7Fh) to -128 (80h).

Indexed Addressing:

Sample mnemonic:	LD A,(IX+20h)

This will load the accumulator with the value pointed to by the register IX plus the offset of 20h.

3.0 A Look At The Emulator:

When the emulator is first started the first screen that you will see is the one shown in figure 5.

This screen is where all assembler mnemonics are entered, if you are not familiar with assembler then do not worry as this will be covered later.

Figure 5 - Text editor

Many of the items along the menu bar should be familiar to you as this program was designed to provide a continuity with other programs.

Starting with the first menu choice, File, should bring up the following sub-menu.

Figure 6 - File sub-menu

This sub-menu offers the standard choices that one would expect from the File menu. You have:

#	New which clears the current document so that you may start afresh.

#	Open so that you may load a previously saved document.

#	Save allows you to save an assembler file.

#	Save As lets you save the current file under a new name.

#	Print so that you may print a file out.

#	Exit terminates the program.

The next sub-menu along is for the edit functions. This should appear as shown in figure 7.

Figure 7 - Edit sub-menu.

From here you may:

#	Undo lets you undo the last change that you made.

#	Cut allows you to remove a block of selected text from the screen. This text is transferred to the clipboard.

#	Copy does the same as Cut but leaves the text in place.

#	Paste will place the text held in the clipboard at the current cursor position.

#	Delete completely erases the selected text.

#	Select All will select all of the available text in the editor.

The next sub-menu is for the search/replace functions. These should appear as shown in figure 8.

Figure 8 - Search/Replace sub-menu

From here you may:

#	Find will let you search for a particular word or phrase.

#	Find Next allows you to search for the next occurrence.

#	Replace lets you search for particular text and replace it with a new selection of text.

The next sub-menu is for the Tools, see figure 9.

Figure 9 - Tools sub-menu

From here you may:

#	Assemble will bring up the assembler control dialog. From there you can assemble your file into Z80 machine code.

#	Z80Runtime is where you execute the machine code program.

The final menu along the top is the help menu. This looks like figure 10.

Figure 10 - Help sub-menu

From here you may:

#	Help tells you to read this manual.

#	About Z80 Emulator tells you about the version and author.

3.01 Assembling:

Selecting the Assemble... button from the Tools menu will cause the current file in the editor to be assembled.

Any problems that are discovered will be flagged at this point and you will end up with a display like the one in figure 11.

Figure 11 - Assembly Failure

All lines that have errors will be stated in the errors box, along with an appropriate error message. You should correct any errors before trying to assemble again. This can be done by either selecting the OK option and correcting the errors by your own memory or by leaving the assemble dialog box active you may change text in the editor and then simply re-assemble. The Re-Assemble button will become active as soon as a change is made to any text in the editor, see figure 12.

Notice that there is progress bar which shows how far there is to go with the current assembly process.

Figure 12 - Active Re-Assemble Button

When all things go well you will have a display as shown in figure 13.

Figure 13 - Successful Assembly

This means that you can now activate the Z80 runtime module. It is this module that provides the interface between you and the core operations of the Z80. To activate the runtime module you will need to select it from the Tools menu.

3.02 Z80Runtime:

Figure 14 - Choice of start positions

When the Z80 Runtime... command is selected from the Tools menu the following dialog box will be displayed. If you have more then one ORG command in your assembler code then you will have a multiple choice of start positions for your program. The ORG command will be discussed later in the manual along with other pre-processor directives. The default position for starting is 0000h, if you have more than one position to choose from then simply double click on the relevant ORG or just select it then select OK.

Once this has been done you will be presented with the runtime module, as shown in figure 15.

Figure 15 - Z80Runtime display

The top of the display shows the register output. From here you can view the before and after status of the Z80 registers, this allows you to see exactly how an instruction affects the data in the CPU.

Figure 16 shows a zoomed shot of the memory display.

Figure 16 - Memory display

From here you can view any of 65536 available memory locations. The sixteen buttons to the right of the display allow you to select a particular 4k block of memory whilst the slide control on the display itself allows you to view a particular location.

The highlighted bar shown in figure 16 is associated with the memory tracking feature. What this means is that when an instruction is executed by the emulator this bar will move to the next instruction to be executed. This allows you to track your programs execution.

At certain times you may wish to keep a watch on a particular location in memory whilst waiting for some action to be performed. To do this simply bring the required location into view in the display window and then simply double click on it with the left mouse button. You will see a message displayed in the message box below stating that the memory lock is active. When an active lock is in place the memory tracking feature will be temporarily disabled. To remove the lock simply double click on the same location or if you wish to move the lock to a new location just repeat the steps required to initiate a lock.

Next to the memory display is the command panel, as shown in figure 17.

Figure 17 - Command panel

This set of controls allows you to perform the following functions:

Stack - By selecting the stack command you will be presented with the data held at the stack pointer and the six positions above it. This will be displayed in the message box as shown in figure 18.

Figure 18 - Stack output

Org(s) - This brings up the dialog box that you saw before the runtime module was activated. You may jump to particular code segments that your program produced from here.

Register - This command brings up the register control dialog box, see figure 19. From here you may alter the value of the Z80 registers.

Figure 19 - Alter registers display

To alter a register simply select the required register and type the new information in. Do not forget that the input is expected in hexadecimal format.

Any errors that you make will be flagged when you press Done.

Halt - This command is used to stop the emulator when it is operating in free run mode.

Single Step - When this box is highlighted the emulator will only execute the next command when the execute button is pushed.

Free Run - This mode is the opposite of single step. By selecting this mode the emulator will run at one of three selectable speeds. The speed may be changed via the Set-Up dialog.

Show Binary - When you wish to see how the registers have been affected by a particular bitwise operation then select this button. The display will be similar to figure 20.

Figure 20 - Binary display output

Execute - This causes the emulator to begin execution. Execution will begin from the current position in memory pointed to by the program counter (PC).

At the bottom of the screen is the messages box. This box is where any information relevant to the execution of a program will be channelled, see figure 21.

Figure 21 - Message output

This box will display information about the current execution, values of operands and any form of exception messages.

To the right of the messages box are the Done! And Set-up buttons, see figure 22.

Figure 22- Done and Setup

By selecting Done! The emulator will exit the runtime mode and fall back to the assembler. If you enter the runtime module again then register values will be reset to zero before execution begins.

Choosing Set-Up causes the following dialog box as in figure 23 to appear.

Figure 23 - Set-Up options

From here you may change the speed at which the emulator works in free run mode or switch off/on the memory tracking.

The speeds for the free run are:

#	Fast 		- 1 Second between execution cycles.

#	Medium 	- 3 Seconds between execution cycles.

#	Slow		- 6 Seconds between execution cycles (default).

4.0 Assembler Directives:

The assembler features pre-processor commands to aid in creating machine code programs.

A pre-processor command does not produce executable code it just makes the assembler perform a task.

Command:	ORG

Syntax:	ORG n

Description:

The number n may be hexadecimal, binary or decimal. By specifying an ORG statement in your code you will cause the code following it to be produced from that point onwards. ORG comes from the word ORIGIN(ATE).

Example:

ORG 1000h

LD A,B	; This code will be produce at location 1000h

Command:	DEFB

Syntax:	DEFB n	DEFB ‘n’

Description:

This command cause the value n to be written to the next location in memory. n may be in hexadecimal, binary of decimal. DEFB means DEFINE BYTE and as such may only register one byte at a time. You may string several together by using a comma as a separator. The pointer to the next valid memory location will be moved to the byte after the statement.

Example:

DEFB ‘a’ , ‘e’ , 100 , 30h , 11111010b

Command:	DEFW

Syntax:	DEFW nn

Description:

DEFW will cause a value which is two bytes long, hence the word, to be written to the next memory location. The value nn may be hexadecimal, binary or decimal. The pointer to the next location in memory will be moved to the next location after the statement.

Example:

DEFW FFFFh

Commmad:	DEFS

Syntax:	DEFS nn

Description:

Causes nn number of bytes to allocated as storage. nn may be hexadecimal, binary or decimal. DEFS means DEFINE STORAGE. The memory pointer will move to the next location at the end of the storage.

Example:

DEFS 100101b

Command:	DEFM

Syntax:	DEFM “Hello World” [,0]

Description:

DEFM is used to store a string of text in memory starting from the current position. You may place a NULL terminator on the end by adding a comma followed by a zero after the string. The NULL terminator is often used as a flag to show that the end of the text string has been reached. DEFM means DEFINE MEMORY.

Example:

DEFM “WELCOME TO THE Z80",0

DEFM “There’s no NULL terminator”

Command:	;

Syntax:	;

Description:

The semi-colon is used to indicate that the following text on the line is a comment.

Example:

LD A,B ; This text will be ignored by the assembler

; and so will this.

Command:	EQU

Syntax:	name EQU nn

Description:

The label name may be any word up to 14 characters in length. All EQU names are turned into upper-case so the assembler will not differentiate between the two different states. EQU means EQUATE and allows you to assign the value nn to the label name. You may have a maximum of 100 EQUATES in your program. nn may be hexadecimal, binary or decimal.

Example:

HAPPY EQU FFFFh

Command:	name:

Syntax:	name:	name

Description:

This provides a label to be used as a jump point in your program. You should use the colon when declaring the labels position and not when you are referencing it. You may have up to 100 LABELS in your program, the assembler will turn all LABELS into upper-case so lowercase will be treated as the same name as an upper-case.

Example:

LD B,20

LOOP:

DJNZ,LOOP

5.0 Exercises:

1]	Simple arithmetic can be achieved on a Z80 using the ADD or ADC command. As you know from the information given on the Z80 registers the A register is normally used for arithmetic.

Try this program:

LD A,10	; just a number

ADD A,30	; add 30 to it

HALT	; in case you’re in free run

What was the result?

What would happen if you changed 30 to 246?

Create a program that takes a value from location 0500h, adds it with the value held at 0501h and then stores the result at location 0502h. You should place the values at 0500h and 0501h before then addition.

Relevant instructions:

LD IX,nnnn	-	This command is used to initialise the X register with a 16 bit value. The immediate addressing mode is used and the command requires 4 bytes.

LD A,(IX+n)	-	As stated earlier this is indexed addressing. The offset value is specified as n.

ADD A,(IX+n)	-	The accumulator added with the value held at the indexed address. ADD is not restricted to indexed addressing.

Answer:

ORG 0500h		; Position to store data

DEFB 100,45	; The data

ORG 0000h		; Start of program

LD IX,0500h	; Set up index register

LD A,(IX+0)	; Load accumulator with first value

ADD A,(IX+1)	; Add second value

LD (IX+2),A	; Store result

HALT		; In case you’re in free run

Was your program similar?

The program above utilised an index register to provide a base point from which the values could be referenced. There are different ways that a program could have solved this problem, try to devise others.

2]	You have seen how 8 bit addition can be achieved, how do you think 32 bit addition could be done, use BCD to solve the following problem?

Add the four byte (32 bit) BCD number stored at locations 0500h - 0503h to the four byte BCD number at locations 0504h - 0507h. Output the result to locations 0508h - 050Bh. In all cases the lower memory location is the LSB and the higher memory location is the MSB.

What is BCD:

BCD is a way of using the two nibbles that make up a byte to represent two decimal numbers. You know that a nibble is four bits long, therefore you can represent a number from 0 - 15. As we are only interested in the numbers 0 - 9 there will have to be some form of adjustment made after any arithmetic is applied. The Z80 comes with an instruction dedicated to this task, it is known as DAA or Decimal Adjust Accumulator.

Relevant instructions:

AND A	-	The logical AND instructions perform a bit by bit AND operation with the accumulator. In this case A is ANDed with itself. Although this seems somewhat strange at first as A will not change you should note that the carry flag will be reset by this.

EX AF,AF’	-	As you know only one set of the alternate register set is active at any one time. This command forces the Z80 to flip between the A and F registers. To exchange the general purpose registers you should use the command EXX.

LD A,(BC)	-	Here indirect addressing is used to load the accumulator with the contents of the memory pointed to by BC. When A is the destination register any of the register pairs may be used to specify the source. If a general purpose register is used then only the HL register pair is valid.

ADC A,(HL) -	Indirect addressing used to add the contents pointed to by HL with the accumulator and the carry flag.

DAA		-	This is used after an arithmetic operation to adjust the value of the accumulator to a valid BCD number. It is normally helpful to specify your BCD number using hexadecimal as the value in hex is a direct representation of what the number is that you want.       Ie - 99 would be 99h where the upper nibble is a value of nine as well as the lower nibble.

INC BC	-	This causes the register pair BC to incremented by 1. Any of the general purpose registers may be specified as well as the register pairs.

DEC A	-	Same rules as above but with the register(s) being decremented by 1.

JR cc		-	The causes a jump relative to the current position. The jump distance is a 2's compliment number, cc is the condition. Using jump relative means that your code can be relocatable.

Answer:

NUMB1	EQU 0500h

NUMB2	EQU 0504h

SUM		EQU 0508h

ORG 0500h

DEFB 34h,0,0,0

DEFB 99h,0,0,0

ORG 0000h		; PROGRAM START POINT

LD BC,NUMB1	; POINT AT FIRST NUMBER

LD HL,NUMB2	; POINT AT SECOND NUMBER

LD DE,SUM	; POINT AT RESULT POSITION

AND A		; CLEAR CARRY FLAG

EX AF,AF’		; SAVE CARRY STATUS

LD A,4		; FOUR RUNS TO COMPLETE

NEXT:		; LABEL

EX AF,AF’		; RECOVER CARRY STATUS

LD A,(BC)		; GET FIRST NUMBER

ADC A,(HL)	; ADD TO SECOND WITH ANY CARRY

DAA			; ADJUST FOR BCD

LD (DE),A		; SAVE RESULT

INC BC		; ADVANCE POINTERS

INC HL

INC DE

EX AF,AF’		; RECOVER NUMBER OF RUNS

DEC A		; ONE LESS RUN TO DO

JR NZ,NEXT	; ANY MORE?

HALT		; END OF PROGRAM

This program made use of some fairly advanced topics. There was the use of EX to swap the values held in the alternate register set with those in the current. This meant that no extra memory was required to store the counter.

Look at how the number is stored, what would happen if there was a carry from the far byte?

Why didn’t we use another register as the counter?

There was the use of DAA to provide the BCD output.

What type of application do you think is best suited to BCD?

The use of JR NZ was used here.

What kind of branch is JR?

How many different modes of address were used in this program?

3]	As the Z80 does not feature any instructions that relate directly to multiplication (excluding a shift left or shift right instruction, this would be the same as x2 or ÷2) how could you multiply two numbers together.

Multiply the byte value held at location 1000h with the byte value held at 1001h. Store the result starting at location 1002h.

8 bit registers can hold the byte values, are they suitable for multiplication?

Relevant instructions:

ADD HL,HL	-	The only time that the Z80 allows 16bit addition is when the value is in the HL register pair. Any of the register pairs may be added to HL.

RLCA		-	Rotate the contents of the accumulator left circular. Bit 7 is rotated into the carry flag as well as bit 0.

DJNZ		-	This causes the B register to be decremented by 1. If the value of B is not zero then a jump relative occurs.

LD (nn),HL		-	Save the HL pair at the address specified by nn. The value is always stored in a low byte then high combination. Therefore n would hold the value of L and n+1 would hold the value of H. Any of the register pairs may be used.

LD L,D		-	Any general purpose register may be copied to another general purpose register.

Answer:

DATA	EQU 1000h ; ADDRESS OF 1ST BYTE

PRDCT	EQU 1002h ; ADDRESS OF PRODUCT

ORG 1000h 	; SET UP NUMBERS

DEFB 10,100

ORG 0000h 	; START OF PROGRAM

LD B,8		; BIT COUNTER

LD HL,DATA	; POINTER TO MULTIPLIER

LD A,(HL)		; GET THE NUMBER

INC HL		; NEXT NUMBER

LD E,(HL)		; E HOLDS MULTIPLICAND

LD D,0		; PREPARE FOR 16BIT ADD

LD L,D		; HL MUST START WITH ZERO

LD H,D

MTPY:

ADD HL,HL	; ROTATE PART PRODUCT LEFT

RLCA		; TEST MULTIPLIER

JR NC,SKIP	; NO ADD IF ZERO

ADD HL,DE	; ADD MULTIPLICAND

SKIP:

DJNZ,MTPY	; DO EIGHT TIMES

LD (PRDCT),HL	; SAVE THE PRODUCT

HALT		; STOP FREE RUN MODE

4]	The following program reads through memory and counts the number of times the value 255 turns up in a particular block of memory. The program starts searching at location 0000h and stops at 00FFh. The number of occurrences is stored in the C register.

No more relevant instructions will be given as you should now look them up for yourself.

START		EQU 0000h

ORG 0000h

DEFS 100

DEFB FFh

DEFS 40

DEFB FFh

ORG 0100h

LD HL,START

LD B,FFh

MAIN:

LD A,(HL)

CP FFh

JR NZ,NEXT

INC C

NEXT:

INC HL

DJNZ,MAIN

HALT

Try and change the number that it searches for to another.

Can you comment the code in the program?

Can you increase the length of the search so that more then 256 locations are searched?